| Data Source | Provider | Type of Data | Variables Available | Temporal Resolution | Coverage | Access | Typical Use in EPF |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Day-Ahead Market Data | European Power Exchange (EPEX SPOT) | Electricity market prices | Day-ahead prices, traded volumes | Hourly | Germany, France, Austria, Switzerland | Exchange / commercial | Primary target variable for most EPF models |
| Intraday Market Data | European Power Exchange (EPEX SPOT) | Electricity trading data | Intraday prices, trades, order book data | 15-min to hourly | Central Europe | Exchange / commercial | Intraday forecasting and trading strategy models |
| Transparency Platform | ENTSO-E | Power system fundamentals | Load, generation by type, cross-border flows, outages | 15-min / hourly | All European markets | Free API | Core explanatory variables in ML forecasting models |
| German TSO Open Data | 50Hertz Transmission GmbH, Amprion, TenneT, TransnetBW | Power system data | Renewable generation, grid load, balancing energy | 15-min | Germany | Free | High-resolution renewable and grid data |
| Open Power System Data (OPSD) | Open Power System Data | Aggregated energy dataset | Electricity prices, load, renewable generation, weather | Hourly | Europe (focus Germany) | Free | Clean benchmark dataset widely used in research |
| Weather Forecast Data | European Centre for Medium-Range Weather Forecasts | Meteorological data | Temperature, wind speed, solar radiation, pressure | Hourly / daily | Global | Subscription / research access | Renewable generation and demand forecasting |
| Climate Reanalysis Data | Copernicus Climate Change Service | Historical weather data | Wind speed, irradiance, temperature | Hourly | Global | Free (research) | Historical weather input for ML models |
| Electricity Futures Data | European Energy Exchange | Power derivatives | Futures prices for power delivery | Daily | Europe | Commercial | Long-term electricity price expectations |
| Fuel Market Data | Intercontinental Exchange | Commodity market data | Gas prices, coal prices | Daily | Global | Commercial | Marginal generation cost predictors |
| Carbon Market Data | European Union Emissions Trading System | Carbon allowance prices | EU ETS CO₂ prices | Daily | Europe | Public / exchange data | Important driver of electricity prices |
| Electricity Load Data | ENTSO-E / TSOs | Demand data | Actual load, load forecasts | Hourly / 15-min | Europe | Free | Key explanatory variable in price models |