Merit Order & Market Clearing Price

A Practical Example with Supply and Demand Curves

# 1. What is the Merit Order?

The Merit Order is the sequence in which power plants are dispatched to meet electricity demand, ranked by their marginal cost of production (variable cost per MWh) in ascending order — from cheapest to most expensive.

In a uniform-price auction (used at EPEX SPOT for the Day-Ahead market), all accepted sellers receive the same price — the Market Clearing Price (MCP) — regardless of their individual bid. This incentivizes truthful bidding at actual marginal costs.

## Typical Merit Order Ranking

| Rank | Technology | Marginal Cost (approx.) | Reason |
| --- | --- | --- | --- |
| 1 | Wind & Solar | ~0 €/MWh | No fuel cost |
| 2 | Nuclear | ~10 €/MWh | Low fuel share |
| 3 | Lignite | ~30–40 €/MWh | Cheap coal, but CO₂ costs |
| 4 | Hard Coal | ~50–70 €/MWh | More expensive coal + CO₂ |
| 5 | Gas (CCGT) | ~70–120 €/MWh | Gas price + CO₂ |
| 6 | Gas (Peaker) | ~120–180 €/MWh | Old, inefficient turbines |
| 7 | Oil | ~200+ €/MWh | Extreme situations only |

# 2. Example: Price Formation with Supply & Demand

The following example demonstrates how the Market Clearing Price is determined for a specific delivery hour (e.g., 14:00 on the following day) through the intersection of supply (sell orders) and demand (buy orders).

## 2.1 Sell Orders (Supply Side) — Sorted Ascending by Price

| Generator | Capacity | Cumulative | Bid Price |
| --- | --- | --- | --- |
| Wind & Solar | 25 GW | 25 GW | 0 €/MWh |
| Nuclear | 8 GW | 33 GW | 10 €/MWh |
| Lignite | 10 GW | 43 GW | 35 €/MWh |
| Hard Coal | 9 GW | 52 GW | 55 €/MWh |
| Gas (CCGT) | 6 GW | 58 GW | 80 €/MWh |
| Gas (Peaker) | 4 GW | 62 GW | 95 €/MWh |
| Oil Reserve | 3 GW | 65 GW | 150 €/MWh |

## 2.2 Buy Orders (Demand Side) — Sorted Descending by Willingness to Pay

| Buyer | Volume | Cumulative | Max. Price |
| --- | --- | --- | --- |
| Baseload Utilities | 30 GW | 30 GW | 200 €/MWh |
| Industrial Consumers | 12 GW | 42 GW | 120 €/MWh |
| Mid-Load Suppliers | 10 GW | 52 GW | 90 €/MWh |
| Flexible Consumers | 5 GW | 57 GW | 70 €/MWh |
| BESS (charging) | 2 GW | 59 GW | 40 €/MWh |
| Pumped Hydro (charging) | 3 GW | 62 GW | 20 €/MWh |

Logic: Baseload utilities must deliver to end customers — they will pay almost any price. Flexible consumers and storage operators drop out when prices are too high.

# 2.3 Finding the Market Clearing Price

The MCP is found where cumulative supply equals cumulative demand at a given price level:

| Price Level | Cumulative Supply | Cumulative Demand | Result |
| --- | --- | --- | --- |
| 0 €/MWh | 25 GW | 62 GW | Demand >> Supply |
| 10 €/MWh | 33 GW | 62 GW | Demand > Supply |
| 35 €/MWh | 43 GW | 59 GW | Demand > Supply |
| 40 €/MWh | 43 GW | 57 GW | Demand > Supply |
| 55 €/MWh | 52 GW | 52 GW | ✅ Equilibrium |
| 80 €/MWh | 58 GW | 42 GW | Supply > Demand |

➡ Market Clearing Price (MCP) = 55 €/MWh at a cleared volume of 52 GW

# 3. Auction Results

## 3.1 Accepted Sellers (Bid ≤ 55 €/MWh)

| Generator | Marginal Cost | Receives | Inframarginal Rent |
| --- | --- | --- | --- |
| Wind & Solar | 0 €/MWh | 55 €/MWh | +55 €/MWh |
| Nuclear | 10 €/MWh | 55 €/MWh | +45 €/MWh |
| Lignite | 35 €/MWh | 55 €/MWh | +20 €/MWh |
| Hard Coal (marginal) | 55 €/MWh | 55 €/MWh | ±0 €/MWh |

The marginal generator (Hard Coal at 55 €/MWh) sets the price and earns zero rent. All cheaper generators earn the difference between the MCP and their marginal cost — this is called the "inframarginal rent."

## 3.2 Accepted Buyers (Bid ≥ 55 €/MWh)

| Buyer | Max. Price | Pays | Consumer Surplus |
| --- | --- | --- | --- |
| Baseload Utilities | 200 €/MWh | 55 €/MWh | 145 €/MWh |
| Industrial Consumers | 120 €/MWh | 55 €/MWh | 65 €/MWh |
| Mid-Load Suppliers | 90 €/MWh | 55 €/MWh | 35 €/MWh |
| Flexible Consumers | 70 €/MWh | 55 €/MWh | 15 €/MWh |

## 3.3 Not Accepted

| Participant | Side | Reason |
| --- | --- | --- |
| Gas (CCGT) | Supply | Bid of 80 €/MWh too expensive — not needed |
| Gas (Peaker) | Supply | Bid of 95 €/MWh too expensive |
| Oil Reserve | Supply | Bid of 150 €/MWh too expensive |
| BESS (charging) | Demand | Willing to pay max. 40 €/MWh — MCP is 55 € → does not charge |
| Pumped Hydro | Demand | Willing to pay max. 20 €/MWh → does not charge |

# 4. Implications for Battery Energy Storage (BESS)

In this scenario, the MCP of 55 €/MWh is too high for the BESS to charge (max. willingness to pay: 40 €/MWh). However, consider a nighttime hour with high wind output and low demand:

| Scenario | MCP | BESS Action | Spread |
| --- | --- | --- | --- |
| Night (high wind, low demand) | 5 €/MWh | Charges at 5 €/MWh | — |
| Afternoon peak (high demand) | 95 €/MWh | Discharges at 95 €/MWh | 90 €/MWh |

The ability to predict these price spreads across delivery hours is the core value proposition of a BESS optimization and forecasting model. The wider and more predictable the spread, the stronger the business case for battery storage arbitrage.

## How BESS Affects the Merit Order

• When BESS discharges (sells): It adds supply to the Merit Order, potentially displacing expensive gas plants → MCP decreases.
• When BESS charges (buys): It adds demand → the demand curve shifts right → a more expensive generator may be needed → MCP increases.
• Net effect: BESS acts as a price-smoothing mechanism — buying in low-price hours, selling in high-price hours, thereby reducing overall price volatility.