WattWise

Feature Selection

List of features used in the research papers (Docs folder) on forecasting day-ahead price

Cecilia:

Biro 2026

Target: day-ahead price in several European countries (hourly)

Features: five data categories used

•         	historical generation from different energy sources (fossil oil, fossil gas, fossil hard coal, fossil brown coal, solar, wind offshore, wind onshore, nuclear, other renewable, etc.)

•         	historical prices

•         	historical cross-border flow data

•         	forecasted demand estimates (load forecast)

•         	forecasted wind and solar generation (wind onshore forecast, wind offshore forecast, solar forecast)

The generation and forecast features are measured in MWh and the price is expressed in EUR/MWh.

Time indicators: Day of Week (1 to 7, Monday to Sunday), Month (1 to 12, January to December), Day of Month (1 to 31), and Day of Year (1 to 366, accounting also for leap years).

Missing data imputation: for Tuesday, Wednesday, Thursday, Friday, or Sunday, if any data are missing we impute values using the same hour's data from the previous day. For Monday and Saturday, we use the same hour's data from the same day in the previous week.

Jasinski 2025

Reviews recent research on electricity price forecasting, collects typical features in Table 4

·        Lagged electricity prices

·        Gas or other fuel prices

·        Demand/load/consumption (lagged, real-time or forecasted)

·        Total generation (actual or forecasted)

·        Generation from combustion-based energy sources and nuclear power (scheduled or forecasted)

·        Generation from non-combustion/direct renewable sources (forecasted)

·        Weather data

·        Temporal data

“In recent years, the use of increasingly complex feature sets has become more common. Lagged, real-time, or forecasted demand/load is by far the most commonly used input variable, appearing in over 60% of the reviewed studies. Weather data and temporal data appear with moderate frequency, which is somewhat surprising given the central role of weather in shaping electricity demand and the common use of temporal features as indirect representations of weather-driven effects.”

Trebbien 2023

Target: day-ahead price in Germany (hourly)

Features:

1.      Power system features (from ENTSO-E)

a.      day-ahead forecasts of load, solar generation, wind generation, the day-ahead total generation and imports and exports.

b.      The features are aggregated for the four control areas of 50Hertz Transmission, Amprion, TenneT and TransnetBW. Wind generation is aggregated from wind on- and offshore generation. Total generation corresponds to the total scheduled generation in the dayahead market. Import and export is aggregated from the cross-border flows between Germany and the neighboring bidding zones

2.      Fuel prices (from ARIVA.DE AG)

a.      Oil prices and natural gas prices

b.      Because both features have a daily time resolution we create a linear interpolation to get an hourly time resolution matching the time sampling of the model

Feature importance (considering years 2017-2019) shows that the most important are 1. day-ahead forecasts of load, 2. day-ahead forecasts of wind generation, 3. day-ahead forecasts of solar generation, 4. Oil price, 5. Day-ahead forecast of import and export, 6. Natural gas price.

Marcjasz 2022

Target: day-ahead price in Germany (hourly)

Features:

·        Past day-ahead prices of the previous three days and one week ago

·        day-ahead (DA) load forecasts, as well as the past values of the previous day and one week ago

·        DA renewable energy sources forecasts (sum of wind offshore, wind onshore and solar generation day-ahead forecasts), as well as the past values of the previous day

·        EU emission allowance prices (most recent closing price)

·        Fuel - coal, oil and natural gas - prices (most recent closing price)

·        Weekday dummies

ChatGPT’s Feature Table

Below is a practical, literature-grounded list of input features suitable for forecasting hourly day-ahead electricity prices in Germany (2022–2025).

Day-Ahead Electricity Price Forecasting — Germany (Hourly, 2022–2025)

| Feature Category | Column Name | Description | Priority Level | Data Source | Notes / Implementation |
| --- | --- | --- | --- | --- | --- |
| TARGET | price_da | German day-ahead electricity price (€/MWh) | Target | European Power Exchange (EPEX SPOT) / Open Power System Data | Forecast next-day hourly prices |
| Price History | price_lag24 | Price same hour previous day | CORE | Open Power System Data | Strongest predictor |
|  | price_lag48 | Price 2 days ago | CORE | OPSD | Stabilizes forecast |
|  | price_lag168 | Price same hour last week | CORE | OPSD | Captures weekly cycle |
|  | price_rolling_mean_24h | Mean price last 24 hours | OPTIONAL | Derived | Trend feature |
|  | price_rolling_std_24h | Price volatility | OPTIONAL | Derived | Regime indicator |
| Demand | load_forecast | German electricity demand forecast (MW) | CORE | ENTSO-E | Fundamental driver |
|  | load_ramp | Hourly change in load | RECOMMENDED | Derived | Captures demand shocks |
| Renewables | wind_forecast | Wind generation forecast (MW) | CORE | ENTSO-E | Major German price driver |
|  | solar_forecast | Solar generation forecast (MW) | CORE | ENTSO-E | Midday price effects |
|  | residual_load | Load − wind − solar | CORE | Derived | Proxy for marginal generation |
|  | wind_ramp | Change in wind forecast | RECOMMENDED | Derived | Volatility signal |
|  | solar_share | Solar / load ratio | OPTIONAL | Derived | Negative-price indicator |
|  | wind_share | Wind / load ratio | OPTIONAL | Derived | Oversupply indicator |
| Weather | temperature | Average temperature Germany | RECOMMENDED | Copernicus Climate Change Service | Demand sensitivity |
| Fuel & Carbon Markets | gas_price_ttf | European natural gas price | RECOMMENDED | Intercontinental Exchange datasets | Crucial for 2022–2025 crisis period |
|  | co2_price | EU ETS carbon allowance price | RECOMMENDED | https://icapcarbonaction.com/en/ets-prices (downloaded) | Affects marginal cost |
| Calendar Effects | hour | Hour of day (0–23) | CORE | Derived | Strong daily seasonality |
|  | weekday | Day of week (0–6) | CORE | Derived | Weekly structure |
|  | is_weekend | Weekend indicator | CORE | Derived | Demand regime |
|  | holiday_flag | German public holiday | RECOMMENDED | Public calendar | Important anomalies |
|  | month | Month index | OPTIONAL | Derived | Seasonal pattern |
|  | sin_hour, cos_hour | Cyclical hour encoding | OPTIONAL | Derived | Helps ML models |

Christoph:

Timespan:

- start 1. June 2023 until today		why: exclude COVID, Russia/Ukraine “spike”, nuclear shut-down (April 2023), renewable production going up sharply

OR

- start 1. January 2020

DA Grosshandelspreis

Last & Verbrauch

• Prognostizierte Gesamtlast (ENTSO-E)

• Historische Last (Lags: t-24h, t-48h, t-168h/Vorwoche)

Stromverbrauch folgt starken Mustern:

• Gestern gleiche Uhrzeit (t-24h) → ähnliches Verbrauchsprofil

• Vor einer Woche (t-168h) → gleicher Wochentag, ähnliches Muster

• Vorwoche Durchschnitt → glättet Ausreißer

Diese Lag-Features helfen dem Modell, Muster zu erkennen, weil der Strompreis stark mit der Nachfrage korreliert.

Renewables Production

• Wind-Prognose (onshore + offshore)

• Solar-Prognose

• Tatsächliche Erzeugung (als Lag-Feature)

Konventionelle Erzeugung

• Geplante Kraftwerksausfälle / Wartungen

• Verfügbare Kapazität (Gas, Kohle, Nuklear)

Weather

• Temperatur (Heizbedarf → Last)

• Windgeschwindigkeit (→ Windeinspeisung)

• Globalstrahlung (→ Solareinspeisung)

• Wolkenbedeckung

Kalender / Zeitfeatures

• Stunde, Wochentag, Monat

• Feiertage / Brückentage (binär)

• Wochenende (binär)

Markt / Commodities

• Gaspreis (TTF Day-Ahead)

• Kohlepreis (API2)

Grenzüberschreitend

• Import/Export-Flüsse (Nachbarländer)

• Preise Nachbarmärkte (FR, NL, AT)

Priorisierung

1. ENTSO-E Daten (Last, Wind, Solar, Preise)

2. Kalenderfeatures

3. Temperatur — Open-Meteo API (kostenlos)

4. Gaspreis — wenn verfügbar, starker Treiber seit 2021

Josip:

Based on google research and using Claude and ChatGPT

features:

- price (target)

- load (electricity, E/MWh)

- wind_offshore

- wind_onshore

- solar (solar production, MWh)

- temperature (average of 4 GE cities)

- hour

- day_of_week

- month

- gas_price

- coal_price

- co2_price

- price_lag_24h

- price_lag_168h (week)

- price_rolling_24h

- price_rolling_168h